Cost Estimation of  Brick Works

1-Size of Bricks in India

Standard Brick Size-

The standard brick size is 190 mm x 90 mm x 90 mm as per the recommendation of BIS. With mortar thickness, the dimension of the brick becomes 200 mm x 100 mm x 100 mm which is also known as the nominal size of the modular brick.

Modular brick means brick along with cement mortar joints, standard modular brick size is 200mm ×100mm ×100mm, volume of one standard modular brick is 0.2m×0.1m×0.1m =0.002m3, so volume of one standard modular brick is 0.002m3

Conventional/Non Modular/Traditional Size-

The commonly adopted nominal size of traditional/ conventional brick is 23 cm x 11.4 cm x 7.0 cm (9”x4 ½ “x2.75”) approximately.

TABLE -B1

| Type of Bricks/ Tiles | Nominal Size mm | Actual Size mm |
| --- | --- | --- |
| Modular Bricks | 200 × 100 × 100 mm | 190 × 90 × 90 mm |
| Modular tile bricks | 200 × 100 × 40 mm | 190 × 90 × 40 mm |
| Non-modular tile bricks | 229 × 114 × 44 mm | 225 × 111 × 44 mm |
| Non-modular bricks | 229 × 114 × 70 mm | 225 × 111 × 70 mm |

2-Quantities

The number of bricks,  quantity of mortar and the labour required for  any brick work depend upon the actual size of bricks, thickness of wall, thickness of joints etc.

Material and labour required for one cubic meter of brickwork –(Average and inclusive of wastage  )

Bricks 500 nos.  per Cubic Meter  (For both IS and 9” brick size)

Mortar (wet) -	0.23 cum for IS bricks

0.25 cum for 9” bricks

Mortar (dry)-     sum total of all the ingredients:

Lime Mortar -	0.325 cu. m for IS bricks

-0.350 cum for 9” bricks

Kankar lime   -	0.275 cum for IS bricks

-0.300 cum for 9” bricks

Mud mortar    -	0.375 cum for IS bricks

-	0.4000 CUM FOR 9” BRICKS

Cement Mortar - Quantities of cement and sand required per cum of brick work:

TABLE-B2

| Mortar Proportion | 1;2 | 1;3 | 1;4 | 1;5 | 1;6 | 1;7 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | For IS Bricks=0.23 CUM Wet Mortar |  |  |  |  |
| Cement in Kg | 156 | 117 | 87 | 71 | 58 | 51 |
| Sand In Cum | 0.220 | 0.243 | 0.243 | 0.243 | 0.243 | 0.243 |
|  |  | For 9” bricks =0.25 cum m wet mortar |  |  |  |  |
| Cement in Kg | 170 | 128 | 95 | 78 | 63 | 55 |
| Sand In Cum | 0.238 | 0.275 | 0.275 | 0.275 | 0.275 | 0.275 |

Labour:- One cubic meter of brick work

TABLE-B3

| Type of Structure | Foundation and plinth |  | Superstructure |  |
| --- | --- | --- | --- | --- |
| Type of Manpower | Mason | Mazdoor | Mason | Mazdoor |
| Numbers | 0.8 | 2 | 0.9 | 3 |

Extra Manpower will be required for Scaffolding etc.

For arch work with uncut bricks, mortar will be 10per cent more and labour 1.75 times than required for simple work and extra for centrings etc.

Honey- comb brickwork requires 1/10th mortar and half the labour of solid work.

For half- brick work, mortar will be 2.5 cu.m for IS bricks and 2.4 cu.m for 9” bricks per 100 sq.m.

3-Costing

Approximate cost  of one cum  of brickwork (9’’ brick size and 1:5 Mortar Proportion)

TABLE-B4

| Description of item (i) | Quantity (ii) | Rate in Rs (iii) | Cost (iv)=(iixiii) | Reference |
| --- | --- | --- | --- | --- |
| No of Bricks per cum | 500 Nos | 7 per brick | 500x7=3500 |  |
| Cement | 78 kg | 400 per bag of 50 Kg | 78x8=624 | Table B2 |
| Sand | 0.275 Cum | 1000 per cum | 0.275x1000=275 | Table B2 |
| Mason | 0.9 per cum | 800 | 0.9x800=720 | Table B3 |
| Mazdoor | 3 per cum | 500 | 3x500=1500 | Table B3 |
| Total |  |  | 6619 |  |
| Add 5% for Misc Heads, Like Supervision, chokidar, scaffolding etc |  |  | 5% 0f 6619=330 |  |
| Grand total per Cum of Brick Work |  |  | Rs 6949/- |  |

NOTE 1-Cost in Table-B4 has been updated on 28-5-2022. Cost of Brick work will vary with variation in cost of Materials. By updating rates of resources in column (iii )in Table -B4, updated cost can be arrived.

NOTE 2- Above cost of brick Work per cum is exclusive of GST & Contractor Profit. Rates of Materials are taken for Tri-city ( Chandigarh).