Cost Estimation of  Plastering Works

1-General

Cement Plaster  - cement plaster shall be 12 mm, 15 mm or 20 mm thick as specified in the item . 6mm plaster is used on Cement Concrete or RCC work.

2-Quantities

Consumption of materials  for Cement plaster on walls per 100 square metres

Table-1

| Proportion of mortar Cement: Sand | 12 mm thick , Quantity-1.44 cu. m wet |  | 15 mm thick , Quantity-1.72 cu. m (wet) |  | 20 mm thick, Quantity 2.24 cu. m-(wet) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cement (Kg) | Sand (cu. m) | Cement (Kg) | Sand (cu. m) | Cement (Kg) | Sand (cu. m) |
| 1:2 | 980 | 1.37 | 1170 | 1.64 | 1525 | 2.14 |
| 1:3 | 735 | 1.55 | 878 | 1.84 | 1142 | 2.40 |
| 1:4 | 546 | 1.55 | 654 | 1.83 | 852 | 2.38 |
| 1:5 | 446 | 1.51 | 534 | 1.82 | 695 | 2.36 |
| 1:6 | 360 | 1.51 | 430 | 1.81 | 560 | 2.36 |
| Extra Mortar | 20 Per cent |  | 15 Per cent |  | 12 Per cent |  |

| Proportion of mortar Cement: Sand | 6 mm thick , Quantity-0.72 cu. m wet |  |
| --- | --- | --- |
|  | Cement (Kg) | Sand (cu. m) |
| 1:2 | 490 | 0.69 |
| 1:3 | 368 | 0.78 |
| 1:4 | 273 | 0.78 |
| 1:5 | 223 | 0.76 |
| 1:6 | 180 | 0.76 |
| Extra Mortar | 25 Per cent |  |

- The extra mortar is required for filling up the joints in brickwork and uneven surfaces, and also for some wastage.

- Where plaster is to be finished with a floating coat of neat cement, take  approximately 100 kg /100sq. metres extra cement.

Consumption of materials for Cement –lime- sand plaster on walls  per 100 sq. metres :

Table-2

| Proportions of mortar Cement : Lime* : Sand | 12 mm thick |  |  | 15 mm thick |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cement kg | Lime cu. m | Sand cu. m | Cement kg | Lime cu. m | Sand cu. m |
| 1:1:6 | 360 | 0.25 | 1.50 | 430 | 0.30 | 1.80 |
| 1:1:7 | 317 | 0.22 | 1.54 | 378 | 0.26 | 1.83 |
| 1:1:8 | 274 | 0.19 | 1.52 | 327 | 0.23 | 1.82 |
| 1:2:9 | 245 | 0.34 | 1.53 | 292 | 0.45 | 1.82 |

Approximate Labour requirement for 100 quare meter plastering; Mason 8, Mazdoor-16

3-Costing

Cost of cement plastering work of 12mm thick having 1:5 proportion per 100 square meter.

Table-3

| Sr No | Description of resources | Quantity | Rate | Cost in Rs | Reference |
| --- | --- | --- | --- | --- | --- |
| 1 | Cement | 446 Kg | 400 per bag=8 per Kg | 3568 |  |
| 2 | Sand | 1.51 cum=53.3 cft | 32 per cft | 1705 |  |
| 3 | Mason | 8 No | 800 | 6400 |  |
| 4 | Mazdoor | 16 No | 500 | 8000 |  |
| 5 | Watering & Scaffolding |  | Rs 2 per Sq feet | 2150 |  |
| 6 | Total |  |  | 21823 |  |
| 7 | Add 20% extra to fill joints |  |  | 4364 |  |
| 8 | Supervision charges @ 3% |  |  | 654 |  |
| 9 | Grand Total |  |  | 26841 |  |

Costing of plastering work comes out Rs 26841 per 100 square meter which is equivalent to  Rs 25.0 per square foot. By adding 20% extra material cost to fill the joints

NOTE 1-Cost in Table-3 has been updated on 28-5-2022. Cost of plastering work will vary with variation in cost of Materials. By updating rates of resources in column (iii )in Table -3, updated cost can be arrived.

NOTE 2- Cost of labour contract varies from Rs 15 to 20 per square foot depending upon location and type of work.

NOTE 3- NOTE 2- Above cost of plastering work per 100 square meter is exclusive of GST & Contractor Profit. Rates of Materials are taken for Tri-city ( Chandigarh).